EXPERIMENT :06

### Aim

To simulate microwave semiconductor devices and analyze the frequency-conversion characteristics of a microwave mixer.

### Objectives

- To study microwave semiconductor device characteristics.

- To simulate the operation of a microwave mixer.

- To observe RF, LO, and IF signals.

- To verify the frequency-conversion process.

Program code

clc;

clear;

close all;

c = 3e8;

Z0 = 50;

fRF = 10e9;

fLO = 9.5e9;

PRF = -10;

PLO = 7;

ZIF = 50;

CL_expected = 8;

fIF = abs(fRF - fLO);

fSUM = fRF + fLO;

PIF = PRF - CL_expected;

PSUM = -42;

Fs = 100e9;

T = 200e-9;

N = round(Fs*T);

t = (0:N-1)/Fs;

VIF = sqrt(2*ZIF*10^((PIF-30)/10));

VSUM = sqrt(2*Z0*10^((PSUM-30)/10));

vIF = VIF*cos(2*pi*fIF*t);

vSUM = VSUM*cos(2*pi*fSUM*t);

vOUT = vIF + vSUM;

Y = fft(vOUT);

A = 2*abs(Y(1:N/2+1))/N;

A(1) = A(1)/2;

f = (0:N/2)*Fs/N;

P = A.^2/(2*Z0);

PdBm = 10*log10(max(P,1e-15))+30;

[~,iIF] = min(abs(f-fIF));

[~,iSUM] = min(abs(f-fSUM));

fIF_sim = f(iIF);

fSUM_sim = f(iSUM);

PIF_sim = PdBm(iIF);

CL_sim = PRF-PIF_sim;

devIF = abs(fIF_sim-fIF)/fIF*100;

devSUM = abs(fSUM_sim-fSUM)/fSUM*100;

devCL = abs(CL_sim-CL_expected)/CL_expected*100;

fprintf('\n');

fprintf('SIMULATION EXPERIMENT 6\n');

fprintf('MICROWAVE MIXER FREQUENCY CONVERSION\n');

fprintf('-------------------------------------\n');

fprintf('RF Frequency          = %.3f GHz\n',fRF/1e9);

fprintf('LO Frequency          = %.3f GHz\n',fLO/1e9);

fprintf('RF Input Power        = %.2f dBm\n',PRF);

fprintf('LO Power              = %.2f dBm\n',PLO);

fprintf('IF Load               = %.1f Ohm\n',ZIF);

fprintf('\n');

fprintf('THEORETICAL RESULTS\n');

fprintf('IF Frequency          = %.3f GHz\n',fIF/1e9);

fprintf('Sum Frequency         = %.3f GHz\n',fSUM/1e9);

fprintf('Assumed Conversion Loss = %.2f dB\n',CL_expected);

fprintf('Expected IF Power     = %.2f dBm\n',PIF);

fprintf('\n');

fprintf('SIMULATED RESULTS\n');

fprintf('IF Frequency          = %.3f GHz\n',fIF_sim/1e9);

fprintf('Sum Frequency         = %.3f GHz\n',fSUM_sim/1e9);

fprintf('IF Output Power       = %.2f dBm\n',PIF_sim);

fprintf('Conversion Loss       = %.2f dB\n',CL_sim);

fprintf('\n');

fprintf('PERCENTAGE DEVIATION\n');

fprintf('IF Frequency Deviation  = %.3f %%\n',devIF);

fprintf('Sum Frequency Deviation = %.3f %%\n',devSUM);

fprintf('Conversion Loss Deviation = %.3f %%\n',devCL);

figure('Color','w','Position',[100 100 1100 750]);

subplot(2,1,1);

plot(f/1e9,PdBm,'b','LineWidth',1.5);

hold on;

plot(fIF_sim/1e9,PIF_sim,'ro', ...

'MarkerFaceColor','r','MarkerSize',7);

plot(fSUM_sim/1e9,PdBm(iSUM),'ko', ...

'MarkerFaceColor','k','MarkerSize',7);

grid on;

xlim([0 25]);

ylim([-100 0]);

xlabel('Frequency (GHz)');

ylabel('Power (dBm)');

title('Microwave Mixer Output Spectrum');

legend('Output Spectrum','IF Component','Sum Component', ...

'Location','best');

text(fIF_sim/1e9+0.3,PIF_sim+4, ...

sprintf('IF = %.1f GHz, %.1f dBm', ...

fIF_sim/1e9,PIF_sim));

text(fSUM_sim/1e9-5,PdBm(iSUM)+5, ...

sprintf('Sum = %.1f GHz',fSUM_sim/1e9));

subplot(2,1,2);

plot(f/1e9,PdBm,'b','LineWidth',1.5);

hold on;

plot(fIF_sim/1e9,PIF_sim,'ro', ...

'MarkerFaceColor','r','MarkerSize',8);

grid on;

xlim([0 2]);

ylim([-100 0]);

xlabel('Frequency (GHz)');

ylabel('Power (dBm)');

title('Intermediate Frequency Output');

text(0.65,PIF_sim+8, ...

sprintf('IF Power = %.1f dBm',PIF_sim));

sgtitle('Simulation Experiment 6: Microwave Mixer Analysis');

figure('Color','w','Position',[150 150 1000 450]);

bar([fIF_sim/1e9 fSUM_sim/1e9]);

set(gca,'XTickLabel',{'Difference Frequency','Sum Frequency'});

ylabel('Frequency (GHz)');

title('Theoretical and Simulated Frequency Components');

grid on;

ylim([0 22]);

text(1,fIF_sim/1e9+0.5, ...

sprintf('%.1f GHz',fIF_sim/1e9), ...

'HorizontalAlignment','center');

text(2,fSUM_sim/1e9+0.5, ...

sprintf('%.1f GHz',fSUM_sim/1e9), ...

'HorizontalAlignment','center');

Output

### Result

The simulation demonstrated the frequency-conversion operation of a microwave mixer. The intermediate frequency was verified